##### CUBIX

##### (An Intelligent alarm clock)

##### Thoery of operations

##### Surendra Maduula 	 Hai Dang hoang	 Dheerajchand Vummidi

##### Introduction:

##### This project is all about making our lives active in the early morning. Now a-days alarm clock is embedded into our life style. People use it extensively. But, there is a problem! Many of us will try to squeeze out few extra minutes of sleep even if the alarm is ringing. We set it to snooze simply to sleep again even if we had to wake up soon. We want to make snoozing alarm clocks difficult. This project implements a new kind of alarm clock with moving capabilities. If you want to turn off the alarm, first you should try to catch that fast moving alarm clock and play a small mind troubling game to turn off the alarm. This makes us to wake up and start the day at an active pace.

##### Implementation and Architecture:

##### This project was implemented on Nexys 4 DDR board. It makes use of the different resources redi;y avlible on the board. It uses Microblaze as the main CPU and will be running Standalone OS for its operation. We have planned well ahead to allocate the resources required for all the differnet modules used in the system. To get a quick idea, I am listing down all the inputs and outputs from our system.

##### Inputs:

- Pushbuttons

- Slide Switches

- Pmod ENC

- Ultrasonic range finder

- Pmod RTCC

##### Outputs:

- Motors

- OLED RGB

- Speaker

##### A better overview can be obtained by looking at he vlovk diagram.

##### This alarm clock has these following features

- Can show the current time, date and week of the day

- Can accept two alarms.

- Can announce the current time

- Can be set to announce the time every hour.

##### Operating instructions:

##### First, when users turn on the alarm clock, they can see a menu with different options such as set date, set alarm etc.

##### These are the steps to be followed to make it work

##### First , turn on the device and select the set time funcitonality from the menu. Now, set the time using push buttons and Pmod enc. Users have to set the current time , date, week day, month and year.  Once they are set, RTCC stores them in its memory and will be perserved even the power is down.

##### Next, user can go back using the Btn L on nexys 4 board. Now, they can go and set the alarm. While setiing the alarm, they are free to use either Pmod ENC or the push buttons. Using the option ‘Difficulty’, they can choose how many times they have to play that game to turn off the alarm clock.

##### When alarm rings up, it fires the sound from the speaker and robot start moving forward. It scans the 180 degree regin before it and moves according to its navigational alarm clock. When user wakes up, he has to go to that clock and press the btn C to start the game. When that is pressed, Cubix knows user is with it and changes the alarm sound to beeping tone and displays the colour game. Now user has to complete the game to turn off the alarm clock. When he completes the game, alarm stops and goes to the menu. Now he can select the ‘Display time’ option to show the current time.

##### Time announcemnt logic:

##### We implemented time announcement functionality using a state machine approach. We used Block RAMs to store the audio files and used on board amplifier to produce the sound back. A sudo state diagrma is given above. It actually takes a 20 bit data for its operation. Its operation can be described in the following two steps.

- Enable the signal Audio_en

- Write the 20 bit value for time announcement

##### Internally, Addr_calc_state shifts this 20 bit value and translates this into start and end address of the sound file. When all the 16 bits (4 digits in decimal-16 bit in Hexa) got processed, It takes the last digit and determines wheather it has to say AM/ PM . It happens continuously so we cant recognize the delay in the process. Finally when the AM/ PM state gets complete, “It would have announced all the time date. Its internal operation can be better understood by looking at the Verilog code.

##### The core logic for the sound production is given below.

##### AUDIO production:

We converted .wav audio files into coe files and stored in block ROMs. This module extracts the audio data from the block ROM and sends out to the PWM wave generator. Then the PWM wave generator sends the PWM signal to the onboard mono audio amplifier. We have downgraded the sampling rate of audio from 44.KHz to 8KHz to save bROMs.

PWM generation:

A PWM signal will be having fixed frequency but changes the width of the pulses. We can use this digital signal to drive a low pass filter to produce the analog voltage whose amplitude is proportional to the width of the input PWM signal. We have used 8Khz audio so we need to generate a PWM wave who’s frequency should be at least 8Khz.So that we will get one new audio pulse at a rate of 8Khz.We have used the below intersective method to store and retrive the audio files.Our audio controller implementation has to modules. Audio_read.V is responsible for reading the Block ROM and storing it in a register. There is another file called Audio_PWM.v it is responsible for the production of the PWM signal. It looks at the digital value stored in the register and produces a PWM pulse in accordance to that. The below figure gives a glimpse of how the method works. The pulse width of the output PWM signal is proportional to the amplitude of the input sine wave.This PWM wave will be given to the sallen key low pass filter it acts a DAC and gives us the smooth audio out on the mono jack.

Ultrasonic range finder:

This sensor is used to detect the obstacles present in the proximity of the Cubix. We implemented an algorithm to make sure that this clock can actually avoid the obstacles and move away from the user simultaneously.

It has a SR-04 ultrasonic sensor. We have to give a 10 us trigger to the ultrasonic sensor first. Then it activates the sensor and it sends an ultrasonic signal. This signal gets back after getting reflected by objects and we measure the high time of the echo pulse. This high time is directly proportional to the distance. We have a servo motor to give the high degree of freedom for the ultrasonic sensor. We rotate it to 40, 90 and 140 degrees and collect the data whether an obstacle is found or not.

This uses a software compass approach to make sure that the alarm clock is moving away from the user. It uses dead reckoning method to calculate the current angle. If the robot takes right turn, it adds 30 degree to the angle and if it takes left it substracts 30 degree from the angle. So, using this data, we avoid taking the same turns again and again and we avoid taking 180 degree turns as it directs the robot towards the user. Its algorithm is shown below.

Pmod RTCC

The Pmod RTCC is a real-time clock/calendar powered by the Microchip MCP79410. Through the I2C  interface, users are able to configure up to two alarms that can be triggered at a wide variety of possible time

- Interfacing with the Pmod

All communications with the device must specify whether to write to the EEPROM or the RTCC register/SRAM, as well as a register address and a flag indicating whether the communication is a read or a write. The PmodRTCC responds to two I2C address. Address ‘1010111’ is used for access to the EEPROM, and address ‘1101111’ is used for access to RTCC registers/SRAM. The PmodRTCC can communicate using I2C via the 8-pin header J2. The device is configured by writing to the registers within the device. The time registers can be set to specific values and a control register sets their functionality.

A full list of registers and their functionality, as well as communication specifications, can be found in the MCP79410 datasheet available at the link reference.

- IP Connection

PmodRTCC in block diagram:

In Nexys4.v, my group uses JC with JC[2] = SCL and JC[3] = SDA

The IP drivers for PmodRTCC, my group takes from Digilent (The link github at reference).  The code below will show how we can set the time at Pmod RTCC in C code: For example if we want to set

9:40:59 (AM) June/09/2017 Monday

And example how we can set alarm 1 for Pmod RTCC: if we want to set 9:45:00 (AM) June/09 Monday

The code below show when Alarm 1 == current time:

- Flow chart

Red line: control bus

Blue line: data bus

This flow chart explains the core logic behind the GUI implemntation in the CUBIX project. We wanted our GUI clean and smooth. So, we took utmost care in every step to make sure that it is user friendly. It has multiple menu option windows and various options to set date, time , alarms. User can use the button present on the Pmod ENC to play the game directly. First, we show a main menu and from that user can select a sub menu to do some task. For example, user can set alarm from the main menu, can set the time and can go to ‘display time’ functionality. It took a lot of effort to make sure that our GUI is not crashing and it has the same control priorities in every menu.

Challenges, possible Enhancemets:

- Challenges:

- I2C interface: Since the devices on the bus do not actually drive the signals high. My group has  trouble to send and to receive data from I2C bus. It consumed 2 valuble days.

Solution: With advice from professor Roy Kravitz, we solve the problem by connecting the pull-up resistor on SCL and SDA pins.

- IP source from Digilent github: There has a problem about connection port PmodRTCC and PmodRTCC_pmod_bridge in verilog code

Solution: My group must modify the verilog code to satisfy the port connection and re-package the IP.

- While integrating all the modules we observed that navigational module is not in sync with other modules. We had only 10 minutes in our hand before the demo starts.

Solution:

We observed it was because of the delay functions. One of the team members used different version of MB_Sleep function. We came to know that even the both modules had the same function name, Their delays were differnet. We had to change many functions to accomidate for the new delay function.

Future possible enhancements:

- Taking the commands through human voice using microphone array and raspberry pi zero

- Integrating temperature sensor and graph it on Pmod OledRGB

- Ability to change alarm ringtones and speaker volume control

- Implementing the project using  RTOS.

References:

https://reference.digilentinc.com/_media/reference/pmod/pmodrtcc/pmodrtcc_rm.pdf

https://github.com/Digilent/vivado-library

http://ww1.microchip.com/downloads/en/DeviceDoc/20002266F.pdf

https://reference.digilentinc.com/_media/reference/pmod/digilent-pmod-interface-specification.pdf

https://reference.digilentinc.com/learn/programmable-logic/tutorials/pmod-ips/start

https://github.com/sburg3/pwm-audio